Replace laptop memory

	

	


	STEP 1

Unlock the battery and remove it from the laptop.

 

	 
	Need spare parts for Satellite A85 laptop? Search here.

	


	STEP 2

Remove two screws securing the hard drive cover and remove the cover.
Remove one screw securing the modem card and the wireless card cover and remove the cover. 

	 
	 

	


	STEP 3

Slide the hard drive left to disconnect it from the system board. Remove the hard drive.
Carefully disconnect the wireless card antenna cables and remove the Wi-Fi card.
Remove two screws securing the modem card. Lift up the card to disconnect it from the system board. Disconnect the modem card cable. 

	 
	 

	


	STEP 4

Remove one screw securing the optical drive. You can push on the metal bracket to slide the drive from the laptop (you can see the bracket on the right from the screw). Remove the optical drive.

	 
	 

	


	STEP 5

Remove all marked screws from the bottom of the laptop.
 

	 
	 

	


	STEP 6

Start removing the keyboard strip with a sharp object and continue with your fingers.

	 
	 

	


	STEP 7

After you unlocked all plastic latches you can remove the keyboard strip.

	 
	

	

	


	STEP 8

In order to remove the keyboard, you have to remove two screws on the top left and right sides.
	[image: image9.jpg]




Laptop Parts Expert

Parts for Toshiba, IBM, HP, Compaq, Dell and other brands.

Free ground shipping on all orders over $79
[image: image10.jpg]=
=
LAPTOPICAL





Find the Best Laptop 

Notebook computer reviews, opinions and news. 

Laptop reviews by brand and model
[image: image11.png]JACKS





Laptop Jacks 

The best source for laptop DC jacks and AC adapters 

Same day shipping service by 5PM EST


	 
	 

	

	


	STEP 9

Lift up the keyboard to get access to the cable. Do not pull the cable! Unlock the connector first.
Now you can pull the flat cable from the connector and remove the keyboard.
	

	 
	 
	

	


	STEP 10

Remove two screws securing LCD hinges on the back of the laptop.
	

	 
	 
	

	


	STEP 11

Disconnect the video cable on the system board. Do not pull wires. Lift up the connector by edges.
Remove the tape securing the wireless antenna cables. Pull the cables through the opening and remove them from the route.
Remove two screws securing the laptop LCD assembly.
	

	 
	 
	

	


	STEP 12

Lift up and remove the LCD assembly.
	

	 
	 
	

	


	STEP 13

Disconnect the switch board cable on the system board.
Disconnect the speaker cable on the system board.
Disconnect the touchpad cable on the system board.
Remove four screws securing the top cover assembly. 
	

	 
	 
	

	


	STEP 14

Separate the top cover assembly and the laptop base using a guitar pick. 
Move the guitar pick along the side to unlock the latches securing the laptop top cover assembly. 
Remove the top cover assembly.
	

	 
	 
	

	


	STEP 15

Only now you can access the laptop memory.
Remove, replace or upgrade it.
I have no idea why they didn't make an opening in the top cover assembly for easy memory access.
	

	 
	
	


