How to remove the LCD screen and inverter board 

	 
	 

	


	STEP 1

Start laptop disassembly with removing the battery.

The LCD bezel ( LCD mask) has four rubber screw seals.
Remove all rubber seals and after that remove four screws securing the LCD bezel.

	 
	 

	


	STEP 2

Start removing the LCD bezel on the side of the display. 

	 
	 

	


	STEP 3

Continue removing the LCD bezel on the top of the display. 

	 
	 

	


	STEP 4

Finally, move to the bottom part of the bezel.

	 
	 

	


	STEP 5

The LCD screnn bezel has been removed.

	 
	 

	


	STEP 6

Remove two screws from the display hinges. 

	 
	 

	


	STEP 7

Now you can access the back side of the LCD screen. 
Disconnect the video cable. 
Carefully unglue the video cable from the screen.

	 
	 

	


	STEP 8

Disconnect cables from both sides of the screen inverter board. 

	 
	 

	


	STEP 9

The LCD screen inverter board has been removed.
You can replace it with a new one if needed.

	 
	 

	


	STEP 10

Remove two screws from each side of the display. 
These screws securing the LCD screen to the display hinges.

	 
	 

	


	STEP 11

The LCD screen has been removed.
You can replace the screen with a new one if needed.

Are you looking for a new LCD screen? Search here.

	 
	 


